- Write a program for error detection using CRC-CCITT (x16+x12+x5+1).

#include<stdio.h>

#include<string.h>

#define N strlen(g)

char t[28],cs[28],g[]="10001000000100001";

inta,e,c;

voidxor(){

for(c = 1;c < N; c++)

cs[c] = (( cs[c] == g[c])?'0':'1');

}

voidcrc(){

for(e=0;e<N;e++)

cs[e]=t[e];

do{

if(cs[0]=='1')

xor();

for(c=0;c<N-1;c++)

cs[c]=cs[c+1];

cs[c]=t[e++];

}while(e<=a+N-1);

}

int main()

{

printf("\nEnter data : ");

scanf("%s",t);

printf("\n----------------------------------------");

printf("\nGeneratng polynomial : %s",g);

a=strlen(t);

for(e=a;e<a+N-1;e++)

t[e]='0';

printf("\n----------------------------------------");

printf("\nModified data is : %s",t);

printf("\n----------------------------------------");

crc();

printf("\n CRC checksum is : %s",cs);

for(e=a;e<a+N-1;e++)

t[e]=cs[e-a];

printf("\n----------------------------------------");

printf("\nFinalcodeword transmitted is : %s",t);

printf("\n----------------------------------------");

printf("\nTest error detection 0(yes) 1(no)? : ");

scanf("%d",&e);

if(e==0)

{

do{

printf("\nEnter the position where error is to be inserted : ");

scanf("%d",&e);

}while(e==0 || e>a+N-1);

t[e-1]=(t[e-1]=='0')?'1':'0';

printf("\n----------------------------------------");

printf("\nErroneous data : %s\n",t);

}

crc();

for(e=0;(e<N-1) && (cs[e]!='1');e++);

if(e<N-1)

{

printf("\n CRC checksum is : %s",cs);

printf("\nError detected\n\n");

}

else

{

printf("\n CRC checksum is : %s",cs);

printf("\nNo error detected\n\n");

}

printf("\n----------------------------------------\n");

return 0;

}

Output  1:

Enter data : 101

----------------------------------------

Generatngpolynomial : 10001000000100001

----------------------------------------

Modified data is : 1010000000000000000

----------------------------------------

CRC checksum is : 0101000010100101

----------------------------------------

Final codeword transmitted is : 1010101000010100101

----------------------------------------

Test error detection 0(yes) 1(no)? : 0

Enter the position where error is to be inserted : 3

---------------------------------------

Erroneous data : 1000101000010100101

CRC checksum is : 0001000000100001

Error detected

----------------------------------------

Output  2:

Enter data : 101

----------------------------------------

Generatngpolynomial : 10001000000100001

----------------------------------------

Modified data is : 1010000000000000000

----------------------------------------

CRC checksum is : 0101000010100101

----------------------------------------

Final codeword transmitted is : 1010101000010100101

----------------------------------------

Test error detection 0(yes) 1(no)? : 1

CRC checksum is : 0000000000000000

No error detected

----------------------------------------

- Write a Program in C/ C++ for hamming code generation for error detection/correction

#include<stdio.h>

void main() {

int data[10];

intdataatrec[10],c,c1,c2,c3,i;

printf("Enter 4 bits of data one by one\n");

scanf("%d",&data[0]);

scanf("%d",&data[1]);

scanf("%d",&data[2]);

scanf("%d",&data[4]);

//Calculation of even parity

data[6]=data[0]^data[2]^data[4];

data[5]=data[0]^data[1]^data[4];

data[3]=data[0]^data[1]^data[2];

printf("\nEncoded data is\n");

for(i=0;i<7;i++)

printf("%d",data[i]);

printf("\n\nEnter received data bits one by one\n");

for(i=0;i<7;i++)

scanf("%d",&dataatrec[i]);

c1=dataatrec[6]^dataatrec[4]^dataatrec[2]^dataatrec[0];

c2=dataatrec[5]^dataatrec[4]^dataatrec[1]^dataatrec[0];

c3=dataatrec[3]^dataatrec[2]^dataatrec[1]^dataatrec[0];

c=c3*4+c2*2+c1 ;

if(c==0) {

printf("\nNo error while transmission of data\n");

}

else {

printf("\nError on position %d",c);

printf("\nData sent : ");

for(i=0;i<7;i++)

printf("%d",data[i]);

printf("\nData received : ");

for(i=0;i<7;i++)

printf("%d",dataatrec[i]);

printf("\nCorrect message is\n");

//if errorneous bit is 0 we complement it else vice versa

if(dataatrec[7-c]==0)

dataatrec[7-c]=1;

else

dataatrec[7-c]=0;

for (i=0;i<7;i++) {

printf("%d",dataatrec[i]);

}

}

}

Output 1:

Enter 4 bits of data one by one

1

0

1

0

Encoded data is

1010010

Enter received data bits one by one

1

0

1

0

0

1

0

No error while transmission of data

Output 2:

Enter 4 bits of data one by one

1

0

1

0

Encoded data is

1010010

Enter received data bits one by one

1

0

1

0

1

1

0

Error on position 3

Data sent : 1010010

Data received : 1010110

Correct message is1010010

Leaky Bucket

#include<stdio.h>

int main(){
    int incoming, outgoing, buck_capacity, n, store = 0;
    printf("Enter bucket capacity, outgoing rate and no of inputs: ");
    scanf("%d %d %d", &buck_capacity, &outgoing, &n);

    while (n != 0) { //loop over total number of inputs
        printf("Enter the number of incoming packets: ");
        scanf("%d", &incoming);
        printf("Incoming packet size %d\n", incoming);
  if((incoming-outgoing) <= (buck_capacity-store)) //it is possible to send without dropping
{
int sent = outgoing>=incoming?incoming:outgoing; //if incoming is more than outgoing, total sent will be outgoing rest buff
if(sent < outgoing && store != 0)  //if incoming<outgoing, we can add values from the store to be sent till op cap is reached
{
int remaining = outgoing-sent;
while(remaining > 0 && store != 0) //keeps adding one to sent until we run out of op cap or nothing left in buff
{
remaining -= 1;
store -= 1;
sent += 1;
}

}
printf("%d packets sent out\n", sent);
if(outgoing<incoming)
{
store = store + (incoming - outgoing);
}
}
else //packets need to be dropped
{
int dropped = (incoming-outgoing)-(buck_capacity-store); //excess packets are the ones that after sending cant be accomodated in buff
printf("%d packets dropped\n", dropped);
store = buck_capacity;
}
printf("%d out of %d space used in the buffer\n", store, buck_capacity);
        n--;
    }
}